Look at this slide: why is DFT so critical for our design?

In a 3D-IC architecture, multiple dies are stacked together vertically. As a result, internal signals become deeply buried and physically unreachable. Tracing these signals usually demands extra pins—but since high-throughput SRAMs already reserve most pins for data I/O, we hit a severe pin bottleneck.

To solve this, the industry established two key standards: IEEE 1838 and IEEE 1687.

To put it simply, think of a 3D-IC as an apartment building. If you want to find someone, you need two things: know which floor they live on, and their specific room number.

In our design, IEEE 1838 acts like the elevator—it routes test signals vertically across different stacked dies. Once you reach the right floor, IEEE 1687 acts like the room address—it routes the signal inside that specific die to target  SRAM module or test signal.

By combining 1838 and 1687, DFT turns a 3D-IC from an unmeasurable black box into a testable chip.

To bring these standards into practice, tools like Siemens Tessent automatically insert these circuit and generate test patterns for the DFT. However, setting them up can be tedious and complex. So, my project aims to streamline this process--let’s jump into my project goals.

So, to solve this problem, here is my project: A 3-step Automation Flow for DFT.

Our ultimate goal is very simple: to minimize manual script editing. Users just fill in some setting files, and the flow handles the rest.

To achieve this, we divide the work into three steps:Step 1: Pipe Clean  Step 2: Build up templates    Step 3: Automation Integration

Let me give you a quick overview of how these three steps connect.

First, Step 1 is Pipe Clean. Before scripting anything, we make sure the manual flow works perfectly across stacked dies using IEEE 1838 and 1687. This gives us a clean, verified reference flow.

With this solid baseline in place, Step 2 is to build up templates. We extract the changing parameters—like design info and TDR settings of a die translate into simple, reusable templates.

Once these templates are ready, Step 3 is Automation Integration. We use Python to read the CSV parameters and execute the entire Tessent flow automatically.

In short, through these three connected steps, we turn a complex manual task into an easy push-button flow.

Now, let’s dive into the details of each step..

Moving to Step 1: Pipe Clean.

In this stage, our main goal is to add Tessent DFT into our team’s design flow. It follows our standard flow, updated with new TCL scripts to support the inserted logic.

Here are some steps worth to notice that Step 01 tells Tessent which structures to insert. After insertion, it extracts ICL and PDL files for pattern generation—which I’ll cover on the next slide. Once verified, the rest of the flow completes synthesis and gate-level simulation

To help the team get familiar with the flow quickly , I documented the entire process into an interactive HTML guide with reference files for each of step.

With this flow fully verified and documented, we now have a reliable process, so let’s move to the next slide to look at the detailed scripts.

This slide shows we use two key TCL scripts to make the DFT process work: insert_tessent.tcl and pattern_retarget.tcl.

In first tcl, after configuring our desired DFT architecture, Tessent inserts the corresponding logic into out raw RTL based on our setup, while automatically extracting the ICL and SDC files.

If you look at the ICL structure on the right, it clearly defines the circuit's ports and interface properties. Having this ICL allows Tessent to run DRC&LVS checks when connecting this block to other circuits, ensuring all connections strictly follow IEEE standards.

Next comes the another key part: pattern_retarget.tcl. Traditionally, to access a specific DFT signal, we had to manually write out the entire protocol, which was very time-consuming and human error. With retargeting, we only focus on our raw RTL behavior—Tessent handles the rest of the 1687 and 1838 flow for us.

We just write simple PDL for each block, and Tessent turns it into full top-level patterns.

In short, with these two technologies, we’ve successfully built an automated flow from DFT insertion to pattern generation.

Although we successfully completed the Tessent setup, these TCL scripts are just too long and complex. Every time the design changes, we have to modify every setting of each tcl manually. So, we integrated the common setup, pulled out the key variables, and simplified everything into a familiar CSV format……. Once these parameters are defined, the rest of the TCL generation is fully automated.

This slide ties our entire automated flow together.

As mentioned earlier, we rely on two main TCL scripts. To make them run seamlessly, we introduced three Python scripts: read_config.py, parse_icl.py, and gen_html_report.py.

- First, read_config.py converts the CSV setup into Tessent-compatible parameters, allowing DFT insertion to run smoothly.

- Next, parse_icl.py analyzes the ICL structure to extract inserted circuit information and specific names, paving the way for PDL to create successful pattern retargeting.

- Finally, after pattern generation and verification, gen_html_report.py compiles the overall results into a visual HTML report.

In short, these Python scripts bridge our user inputs, Tessent execution, and result visualization into one complete pipeline.